Bluetooth Classic ‘Message Access Profile’

How to obtain templates for correct MAP OBEX objects

# The Problem

# Bluetooth SIG provides several specifications, including:

# MAP_v1.4.2.pdf

# GOEP_v2.1.1.pdf

# Test specification (MAP.TS.p14.pdf)

# Authoritative test tool PTS (v8.6.x currently)

# These documents provide the necessary definitions for OBEX body objects (vCard, XML, etc.) in BNF and/or DTD form, which are essential for creating compliant applications or devices with Bluetooth Classic Message Access Profile v1.4.2.

# However, creating these OBEX objects to pass all relevant PTS test cases is often a cumbersome and error-prone trial-and-error process. Feedback from PTS is typically minimal, and the tool tends to hang or crash, adding to the challenge.

# The Solution

After investigation, it was discovered that PTS compares the body-objects of incoming messages with templates stored in its working directory. These templates can be found in the following directory:

C:\Users\xyz\AppData\Roaming\Bluetooth SIG\Bluetooth PTS 8.6.0.6\install\5C26A5D\Data\Others\OBEX\MAP

With this discovery, the implementation and test cycle typically follows these steps:

- Select the Next Test Case in PTS

Example:  “MAP/MSE/MMN/BV-11-C”

- Identify the Expected Body Object Type in the Description

Example: 'ParticipantChatStateChanged' in the description “Verify that the MSE sends a 'ParticipantChatStateChanged' event-report when the chat-state of a participant is changed.”

- Search the PTS template directory

Example: Search for “Chat” in the directory C:\Users\xyz\AppData\Roaming\Bluetooth SIG\Bluetooth PTS 8.6.0.6\install\5C26A5D\Data\Others\OBEX\MAP\put\ParticipantChatStateChanged_event_report_1_2

<MAP-event-report version = "1.2">

<event type = "ParticipantChatStateChanged" sender_name="PTS" conversation_id=”3909231965” last_activity="20160625T133700" chat_state="1" participant_uci=”skype:beastmode2”/>

</MAP-event-report>

- Use the File’s Contents as the Template

This file’s contents can now be used as the template for the OBEX body-object required for being compliant with the specs and passing the PTS test cases.